BMS Cleaning Philosophy

Our cleaning philosophy is that upon arrival at the designated cleaning facility (1605 Waxwing) from receipt of the customer the unit shall be cleaned using the following standard procedures. No rental unit shall enter storage the offsite storage facility unless it has been steam cleaned and/or disinfected using specific procedures and cleaners listed below*.

Rented Equipment Cleaning Requirements

- Newly purchased standard hospital beds mattresses are covered with plastic covers upon receipt at BMS.

- All metal parts of hospital bed rental unit are to be disinfected and/or cleaned using procedures below

- Non metal cloth, fabric or leather components of rental units shall be disinfected and/or steam cleaned

- Luxury Bed Cloth Mattress covers shall be machine washed and dried before being rented

- Standard Bed Plastic Mattress covers shall be machine washed and air dried prior to subsequent ren

Steam Cleaning Procedures - Fabric and Non Metal Exposed Surfaces

Luxury Full & Semi Electric Hospital Beds and Mattresses and Lift Chairs

- Locate steam cleaner - Shark Steam Cleaner - Model #SC630D , 1500 Watts or equivalent

- Steam clean using Pages 8 & 9 of the Model #SC630D equipment manual

Hospital Bed Cleaning/Disinfecting Procedures

Standard Hospital Beds (Metal Parts only) - Full & Semi Electric:

- Wear gloves, face mask, with protection goggles when handling or cleaning rental beds.

- Spray bed frame lightly with disinfectant.

- Leave spray on for no less than 10 minutes

- Wipe down with clean dry cloths

- Place in designated area ready for rental.

- Update the cleaning log at Bradford Medical Supply  - Equipment Cleaning Log - 2015

Luxury Hospital Beds - Full & Semi Electric:

- Wear gloves, face mask, with protection goggles when handling or cleaning rental beds.

- Spray bed frame pedestal cover lightly with disinfectant.

- Leave spray on for no less than 10 minutes

- Steam Clean seats cloth, plastic or leather surfaces as required using referenced Steam Cleaning Procedure

- Place in designated area ready for rental.

- Update the cleaning log at Bradford Medical Supply  - Equipment Cleaning Log - 2015

Hospital Bed Mattress Cover Cleaning/Disinfecting Procedures

Standard Hospital Bed zippered mattress cover:

- Wash plastic mattress cover for a minimum of 10 minutes using a hot water  heavy duty cycle setting

- Air dry mattress cover

- Insert cleaned mattress in air dried plastic mattress cover

- Place in designated area ready for rental.

- Update the cleaning log at Bradford Medical Supply  - Equipment Cleaning Log - 2015

Luxury Hospital Bed waterproof mattress cover:

- Wash waterproof mattress cover for a minimum of 10 minutes using a hot water  heavy duty cycle setting

- Dry cover using a high temperature setting

- Install clean cover on the luxury 10” innerspring mattress

- Place in designated area ready for rental.

- Update the cleaning log at Bradford Medical Supply  - Equipment Cleaning Log - 2015

Hospital Bed Mattress Cleaning/Disinfecting Procedures

Standard Hospital Bed mattress:

- Wear gloves, face mask, with protection goggles when handling or cleaning rental beds.

- Spray plastic 7” mattress lightly with disinfectant.

- Leave spray on for no less than 10 minutes

- Wipe down with clean dry cloths

- Steam Clean seats cloth, plastic or leather surfaces as required using referenced Steam Cleaning Procedure

- Allow mattress to dry completely (4 hour minimum)

- Place in designated area ready for rental.

- Update the cleaning log at Bradford Medical Supply  - Equipment Cleaning Log - 2015

Luxury Hospital Bed mattress:

- Wear gloves, face mask, with protection goggles when handling or cleaning rental beds.

- Spray bed frame pedestal cover lightly with disinfectant.

- Leave spray on for no less than 10 minutes

- Wash waterproof mattress cover on heavy duty cycle using hot water

- Place in designated area ready for rental.

- Update the cleaning log at Bradford Medical Supply  - Equipment Cleaning Log - 2015

All other Rental Unit Cleaning Procedures

The cleaning procedure applies to all of the rental equipment listed below:

Power Wheelchairs		Walkers 		Knee Walker 		Ramps

Mobility Chairs		Trapeze 		Wheelchairs

Mobility Scooters		Lift Chairs 		Transport Chairs

- Spray all exposed metal parts used by rental clients (e.g., handlebars, joysticks and all other exposed metal parts) lightly with disinfectant.

- Leave spray on exposed metal parts for no less than 10 minutes

- Wipe exposed metal parts down with clean dry cloths

- Locate steam cleaner - Shark Steam Cleaner - Model #SC630D , 1500 Watts or equivalent

- Steam clean using Pages 8 & 9 of the Model #SC630D equipment manual

- Place in designated area ready for rental.

- Update the cleaning log at Bradford Medical Supply  - Equipment Cleaning Log - 2015

* Required Disinfectant or equivalent used:

​

https://www.medline.com/product/Virex-TB-Hospital-Grade-Quaternary-Disinfectant/Germicides-Disinfectants/Z05-PF00617

Virex Tb is a ready-to-use quaternary-based hospital grade disinfectant that provides excellent cleaning and deodorizing in one step. Disinfects in 3 minutes, Bactericide, tubercolocide, virucide, fungicide. Kills norovirus, VRE and MRSA. Meets blood borne pathogen standards for decontaminating blood and body fluids. Colorless with a lemon scent. (No mixing or dilution required)

http://www2.hawaii.gov//hdoa/labels/8017.24_2016.pdf